Developed for industrial applications, they are used to avoid collisions, measure the level of liquids and solids, profile conveyor belts, detect proximity, position and monitor equipment, or even altimetry applications. Variation in material characteristics or difference in environmental conditions can be a contributing factor to a particular sensor not functioning properly.

LDS lasers are class I are low power lasers (< 1 mW), visible light are absolutely safe for the eyes and skin

Measurable objects:   , surfaces of natural objects, metallic non-reflective, wood, plastic, coal,

Non-measurable: mirrors and transparent objects

Environmental Limitations 	Surfaces with very high direct sunlight

Resolution 			0.1mm

Ripetibilita 			±0.3mm（5m），±0.5mm（20m），±0.8mm（50m）

Response time		100ms

Laser Type			640nm red laser, Class 1

Tipi di misura 		Power on automatic measurement/command measurement

Consume 			< 1W

Impedance			≤500Ω

Input voltage			6-32VDC

Communication mode	485,232

Working temperature	-25℃--65℃

Dimensions 			60mm x 61mm x 30mm 150g

IP 			IP68 Grade

Error value specification

Code	Fault cause	solution

03	The temperature is too low	warming

07	Incident light intensity	Reduced incident light influence

04	Temperature is too high	hypothermia

08	Signal is too weak	The reflector is not enough, please use the reflector

09	Signal is too strong	Reduce the reflection of the object under test

15	Vibrate too fast	Reduce the vibration of the measured object

18	Too close	Increase measuring distance

19	Out of range Things move too fast	Reduce measuring distance

Reduce the movement speed of the measured object